PART 1 GENERAL

Furnish all necessary materials, labor, and equipment for the complete installation of the aluminum sliding

doors, frames, and hardware, as shown on the drawings and specified herein.

1.01 SUMMARY

A.	Section includes: Aluminum Sliding Doors, including:

1.   YKK AP Series YSD 400 Heavy Duty Sliding Doors.

B.	Related Sections:

1.	Sealants:  Refer to Division 7 Joint Treatment Section for sealant requirements.

2.	Glass and Glazing:  Refer to Division 8 Glass and Glazing Section for glass and glazing

requirements.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 41 13 Aluminum-Framed Entrances and Storefronts.

b.	Section 08 51 13 Aluminum Windows.

c.	Section 08 44 13 Glazed Aluminum Curtain Walls.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02 TEST AND PERFORMANCE REQUIREMENTS

A.	All test unit sizes and configurations shall conform to the minimum sizes in accordance with; AAMA

101/I.S./NAFS-02, with a performance class of AW, performance grade 60. They shall also comply with

Florida High Velocity Hurricane Zone (HVHZ) Protocols, ASTM E 1886, ASTM E 1996, and meet all

requirements of TAS 201, TAS 202, and TAS 203.	They shall also comply with the following specific

performance requirements indicated.

1.	Air Infiltration:  Completed sliding doors shall have 0.30 CFM/FT² maximum allowable infiltration

when tested in accordance with ASTM E 283 at a differential static pressure of 1.57psf 75 Pa).

2.	Water Infiltration:  There shall be no uncontrolled water leakage when tested in accordance with

ASTM E 331 at a static pressure of 9 psf  (431 Pa) for low sill frames and 15 psf (718 Pa) for high

sill frames.

3.	Uniform Load Structural:  There shall be no damage to fasteners, hardware, accessories, or any other

damage that would render the sliding door inoperable when tested in accordance with ASTM E 330

at a differential static pressure of 60.0 psf positive and negative.

4.	Forced Entry Resistance:  Sliding doors shall meet all test requirements of AAMA 1304.

5.	Deglazing:  Sliding doors shall meet all test requirements of ASTM E 987 and NAFS (5.3.6.2).

6. 	Thermal Performance:  When tested in accordance with AAMA 507:

a. 	Thermal Transmittance U Value:  0.54 BTU/HR/FT²/°F or less.

8.	Acoustical Performance: When tested in accordance with AAMA 1801, the Sound Transmission

Class (STC), and Outdoor–Indoor Transmission Class (OITC) shall not be less than:

a. 31 STC and 27 OITC for annealed glazing.

b. 33 STC and 30 OITC for laminated glazing.

9.  Life Cycle Testing: When tested in accordance with AAMA 910, there shall be no damage to

fasteners, hardware parts, or any other damage that would cause the specimen to be inoperable.

Resistance to air leakage and water penetration resistance test results shall not exceed the gateway

performance.

Note:  Performance based on lab testing and will vary by configuration and glass type; contact YKK AP engineering for job specific analysis at higher performance levels.

1.03 SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with "Conditions of

the Contract" and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in "Conditions of the Contract."

B.	Product Data:  Submit product data for each type sliding door series specified.

C. Substitutions: Whenever substitute products are to be considered, supporting technical data, samples,

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

D.	Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors and textures.

E.	Samples:  Submit verification samples for colors on actual aluminum substrates indicating full color

range expected in installed system.

F.	Quality Assurance / Control Submittals:

1. 	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

2. 	Installer Qualification Data:  Submit installer qualification data.

G. Closeout Submittals:

1. 	Warranty:  Submit warranty documents specified herein.

2. 	Project Record Documents:  Submit project record documents for installed materials in accordance

with Division 1 Project Closeout (Project Record Documents) Section.

1.04 QUALITY ASSURANCE

A.	Qualifications:

1.	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner's and Architect's acceptance of finish color,

and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation: Mock-up may be incorporated into final construction upon Owner's approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer's installation instructions, and manufacturer's warranty requirements.

1.05 PROJECT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings. Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

It is the intent of this specification to have a single source responsibility for the supply of the aluminum

sliding doors and frames on this project.  Any deviation from the acceptable manufacturers listed below

must be approved in writing by the architect at least ten (10) days prior to bid date.

2.01 MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers:	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

1.	AAMA Designation:  AW-60.

2.	Description:  YKK YSD 400 Series Heavy Duty Sliding Doors shall be extruded aluminum with overall frame depth 4-1/16" (103mm) not including receptors or heavy interlock stiles; Double screw butt type frame and panel construction; Factory assembled.

3.  	Configuration:  The sliding doors shall be OX, XX, XO, OXO, or OXXO.

4. 	Glazing:  Each panel shall be channel glazed using a marine type wrap around EPDM gasket; 1" insulating units; Factory or bench glazed.

2.02 MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 Aluminum Alloy.

B.	Aluminum Sheet:

1.	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050" (1.27 mm)

minimum thickness.

2.	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080" (1.95 mm)

minimum thickness.

2.03 ACCESSORIES

A.	Manufacturer's Standard Accessories:

1.	Hardware:  Standard adjustable stainless steel ball bearing rollers, all steel mortise lock with

laminated steel hookbolt, red oak interior pull handle on aluminum alloy and high impact thermal

plastic base, and ZAMAC alloy die cast exterior pull handle.

2.	Fasteners:  All fasteners shall be AISI 300 series (except for self-drilling, which are to be series 400)

stainless steel.

3. 	Sealant:  Non-skinning type, AAMA 803.3

4. 	Glazing:  Marine type wrap around gaskets. EPDM in accordance with ASTM C864. Color Black.

2.04	FABRICATION

A.	Fabricate frame using two screws and adhesive backed gaskets at corner joints. Fixed and sliding panels

shall have over lapped joints, sealed, and dual screw/spline joinery for extra strength and weatherability.

Sliding panels to have two tandem wheel assemblies rolling on a roll formed stainless steel track cover.

Sliding panels shall not be removable when in a locked position.

1.	Hardware:  Drill and cut to template for hardware. Reinforce frames and door stiles to receive

hardware in accordance with manufacturer's recommendations.

2.06	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE		DESCRIPTION

YS1N*		Clear Anodized Plus®

YH3N		Champagne Anodized Plus®

YB1N		Medium Bronze Anodized Plus®

YB5N*		Dark Bronze Anodized Plus®

YK1N*		Black Anodized Plus®

YW3N		White Anodized Plus®

M				Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612-02. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c.	The anodized coating shall comply with all of the requirements of AAMA 612-02: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1.	Type Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by Solvay Solexis,

fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in accordance with

YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors: Selected by Architect from the following:

a.	Standard coating color charts.

b.	Custom coating color charts.

c.	Color Name and Number:

D.	Finishes Testing:

1.	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh; Do not clean area further.

2. 	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01 MANUFACTURER'S INSTRUCTIONS / RECOMMENDATIONS

A.	Compliance:  Comply with manufacturer's product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation instructions are available

at www.ykkap.com.

3.02 EXAMINATION

A.	Site Verification of Conditions:  Verify conditions (which have been previously installed under other

sections) are acceptable for product installation in accordance with manufacturer's instructions.

1. 	Verify location of perimeter fasteners and block-outs are in accordance with shop drawings.

3.03 PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

1.  	Aluminum Surface Protection:  Protect aluminum surfaces from contact with lime, mortar, cement,

acids, and other harmful contaminants.

3.04 INSTALLATION

A	General:  Install manufacturer's system in accordance with shop drawings, and within specified

tolerances.

3.05 FIELD QUALITY CONTROL

A.	Manufacturer's Field Services:  Upon request, provide manufacturer's field service consisting of site visit

for inspection of product installation in accordance with manufacturer's instructions.

B. 	Field Test:  Conduct field test to determine watertightness of sliding doors. Conduct test in accordance

with AAMA 502.2 at locations selected by Architect.

3.06 ADJUSTING AND CLEANING

A.	Adjusting:  Adjust operating items in accordance with manufacturer's recommendations.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer's

instructions prior to owner's acceptance, and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product's finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.