PART 1 GENERAL

Furnish all necessary materials, labor, and equipment for the complete installation of the aluminum

sliding doors, frames, and hardware, as shown on the drawings and specified herein.

1.01	SUMMARY

A.	Section includes: Aluminum Sliding Doors, including:

1.   YKK AP Series YSD 700 H Impact Resistant Heavy Commercial Sliding Doors.

B.	Related Sections:

1.	Sealants:  Refer to Division 7 Joint Treatment Section for sealant requirements.

2.	Glass and Glazing:  Refer to Division 8 Glass and Glazing Section for glass and glazing

requirements.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 41 13 Aluminum-Framed Entrances and Storefronts.

b.	Section 08 44 13 Glazed Aluminum Curtain Wall.

c.	Section 08 44 33 Sloped Glazing Assemblies.

d.	Section 08 51 13 Aluminum Windows.

1.02	TEST AND PERFORMANCE REQUIREMENTS

A.	All test unit sizes and configurations shall conform to the minimum sizes in accordance with AAMA 101/I.S./NAFS-02, with a performance class of HC, performance grade 120, meet all requirements of High Velocity Hurricane Zone (HVHZ) Code Protocols TAS 201, TAS 202, and TAS 203 and comply with the following specific performance requirements indicated.

1.	Air Infiltration:  Sliding doors shall have 0.30 CFM/FT² maximum allowable infiltration when tested

in accordance with ASTM E 283 and TAS 202 at a differential static pressure of 6.24 psf (299 Pa).

2.	Water Infiltration:  There shall be no uncontrolled water leakage when tested in accordance with

ASTM E 331, at a static pressure of :

a. 15 psf (718 Pa) for monolithic glass.

b. 20 psf (958 Pa) for insulating glass.

3.	Static Load:  There shall be no damage to fasteners, hardware, accessories, or any other damage that

would render the sliding door inoperable when tested in accordance with ASTM E 330 and TAS 202

at a differential static pressure of 120 psf positive and negative

4.	Large & Small Missile Impact:  There shall be no signs of penetration, rupture, or opening after the

impact test when tested in accordance with ASTM E 1886/1996, SSTD 12-99 and TAS 201.

5.	Cyclic Load:  Test to be done upon completion of missile impact test. There shall be no damage to

fasteners, hardware, accessories, or any other damage that would render the sliding door inoperable

when tested in accordance with ASTM E 1886/1996 and TAS 203.

6. 	Thermal Performance:  When tested in accordance with AAMA 1503 and AAMA 507:

a. 	Condensation Resistance Factor (CRF):  A minimum of 21.

b. 	Thermal Transmittance U Value:  0.61 BTU/HR/FT²/°F or less.

7.	Acoustical Performance: When tested in accordance with AAMA 1801 the Sound Transmission

Class (STC), and Outdoor–Indoor Transmission Class (OITC) shall not be less than:

a. 30 STC and 26 OITC for annealed glass.

b. 30 STC and 27 OITC for laminated glass.

8.	Life Cycle Testing: When tested in accordance with AAMA 910, there shall be no damage to

fasteners, hardware parts, or any other damage that would cause the specimen to be inoperable.

Resistance to air leakage and water penetration resistance test results shall not exceed the gateway

performance.

9.	Forced Entry Resistance:  Sliding doors shall be tested in accordance with ASTM F 842 & TAS 202

and meet the requirements of performance grade 10.

10.	Deglazing:  Sliding doors shall meet all test requirements of ASTM E 987 and NAFS (5.3.6.2).

Note:  Performance based on lab testing and will vary by configuration and glass type; contact YKK AP engineering for job specific analysis at higher performance levels.

1.03	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with "Conditions of

the Contract" and Division 1 Submittals Section. Product data, shop drawings, samples, and similar

submittals are defined in "Conditions of the Contract".

B.	Product Data:  Submit complete product data including installation instructions.

C. Substitutions: Whenever substitute products are to be considered, supporting technical data, samples,

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

D.	Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors, and hardware.

E.	Samples:  Submit verification samples for colors.  Minimum 2 ½ inch by 3 inch (61mm by 73mm)

samples on actual aluminum substrates indicating full color range expected in installed system.

- Quality Assurance / Control Submittals:

1.	Test Reports:   Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

2.	Project Record Documents:  Submit project record documents, including operation and maintenance

data for installed materials in accordance with Division 1 Project Close-out (Project Record

Documents) Section.

1.04	QUALITY ASSURANCE

A.	Qualifications:

1.	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner's and Architect's acceptance of finish color,

and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation: Mock-up may be incorporated into final construction upon Owner's approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer's installation instructions, and manufacturer's warranty requirements.

1.05	PRODUCT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings.  Coordinate field measurements and fabrication

schedule with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01	MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: 	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

1.	Sliding Doors:  YKK AP YSD 700 H Impact Resistant Heavy Commercial Sliding Doors.

B.	Sliding Doors:

1.	AAMA Designation:  HC-120.

2.  	Description:  YKK YSD 700 H Series Sliding Doors shall be extruded aluminum with overall frame

depth of 7" (177.8mm) not including interlock stiles; Horizontal frame members run through square

cut vertical members, butted and mechanically fastened with stainless steel screws; Vertical panel

members run through notched top and bottom rails, butted and mechanically fastened with two

stainless steel screws per joint; Standard shipped knocked-down or optional pre-glazed sliding

panels.

3.  	Configuration:  The sliding doors shall be OX, XO, OXO, or OXXO.

4. 	Glazing (Contact YKK AP for approved glass types):

a.	Non-Impact & Small Missile: Sliding panel(s) shall be channel glazed using a marine type wrap

around EPDM glazing gasket; Fixed panel(s) shall be glazed with standard glazing stops and

EPDM wedge gasket on the exterior and EPDM sponge gaskets on the interior; 9/16" monolithic

or 1" to 1-3/16" insulating units.

2.02 	MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 and 6063-T6 Aluminum Alloys.

B.	Aluminum Sheet:

1.	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050" (1.27 mm)

minimum thickness.

2.	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080" (1.95 mm)

minimum thickness.

2.03	ACCESSORIES

A.	Manufacturer's Standard Accessories:

1. 	Hardware:  Standard adjustable stainless steel ball bearing rollers, stainless steel track cover, MS

Lock with all steel case and laminated steel hookbolt (1 1/2" backset), 1" aluminum tubular pull

handles, or integral extruded pull handle depending on configuration; Optional thumbturn cylinder

and aluminum cylinder trim ring.

2.  	Fasteners: All fasteners to be AISI 300 series (except for self-drilling which are to be AISI 400

series) stainless steel.

3.  	Sealant:  Non-skinning type, AAMA 803.3.

2.04	FABRICATION

A.	Fabricate frame using three screws per joint at jambs and six screws per joint at mullions; all joints sealed. Sliding panels shall have overlapped joints, sealed, and dual screw/spline joinery for extra strength and weatherability. Each sliding panel to have two stainless steel tandem roller assemblies rolling on a roll-formed stainless steel track cover. Sliding panels shall not be removable when in a locked position.

1.  	Hardware:  Attach pull handles as instructed in YKK AP YSD 700 installation manual. Attach

optional hardware per manufacturer's instructions.

2.05	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE		DESCRIPTION

YS1N*		Clear Anodized Plus®

YH3N		Champagne Anodized Plus®

YB1N		Medium Bronze Anodized Plus®

YB5N*		Dark Bronze Anodized Plus®

YK1N*		Black Anodized Plus®

YW3N		White Anodized Plus®

M				Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1.	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with the

requirements of AAMA 612-02. Aluminum extrusions shall be produced from quality controlled billets

meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612-02: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1. 	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors:  Selected by Architect from the following:

a.	Standard coating color charts.

b. 	Custom coating color charts.

c. 	Color Name and Number:

D.	Finishes Testing:

1.	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh. Do not clean area further.

- Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01	MANUFACTURER'S INSTRUCTIONS / RECOMMENDATIONS

A. Compliance:  Comply with manufacturer's product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation instructions are available

at www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify conditions (which have been previously installed under other

sections) are acceptable for product installation in accordance with manufacturer's instructions.

1.	Verify location of perimeter fasteners and block-outs are in accordance with shop drawings.

3.03	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04	INSTALLATION

A.	General:  Install manufacturer's system in accordance with shop drawings, and within specified

tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials.

2. 	Shim and brace sliding door frame before anchoring to structure.

3.	Completed sliding doors must allow water to be wept to the exterior; Verify weep holes are open.

4. 	Seal metal to metal window system joints using sealant recommended by system manufacturer.

3.05	FIELD QUALITY CONTROL

A.	Manufacturer's Field Services:  Upon request, provide manufacturer's field service consisting of site visit

for inspection of product installation in accordance with manufacturer's instructions.

B.	Field Test:  Conduct field test to determine watertightness of sliding door. Conduct test in accordance

with AAMA 502-02.

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust sliding doors for operation in accordance with manufacturer's recommendations.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer’s instructions prior to Owner's acceptance, and remove construction debris from project site. Legally dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product's finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.