PART 1 GENERAL

Furnish all necessary materials, labor, and equipment for the complete installation of the aluminum terrace doors, frames, and hardware, as shown on the drawings and specified herein.

1.01 SUMMARY

A.	Section includes: Aluminum Doors and Frames

1.   YKK AP Series YTD 350 T MegaTherm® Architectural Terrace Doors.

B.	Related Sections:

1.	Sealants:  Refer to Section 07 90 00 Joint Protection for sealant requirements.

2.	Glass and Glazing:  Refer to Section 08 80 00 Glazing for glass and glazing requirements.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 11 16 Aluminum Doors and Frames.

b.	Section 08 41 13 Aluminum-Framed Entrances and Storefronts.

c.	Section 08 51 13 Aluminum Windows.

d.	Section 08 44 13 Glazed Aluminum Curtain Walls.

e.	Section 08 44 33 Sloped Glazing Assemblies.

1.02 	TEST AND PERFORMANCE REQUIREMENTS

A.	All test unit sizes and configurations shall conform to the minimum sizes in accordance with

AAMA/WDMA/CSA/101 I.S.2/A440-05.

B.	Performance Requirements:  Architectural terrace doors shall conform to all AAMA/WDMA/CSA/101

I.S.2/A440-05 requirements for the door type and comply with the following specific performance

requirements indicated.

1.	Air Infiltration:  Architectural terrace doors shall have 0.10 CFM/FT² maximum allowable

infiltration when tested in accordance with ASTM E 283 and AAMA 101 (5.3.2) at a differential

static pressure of 6.24 psf (300 Pa).

2.	Water Infiltration:  There shall be no uncontrolled water leakage when tested in accordance with

ASTM E 331, at a static pressure of:

a.	15 psf (720 Pa) for outswing single and pair.

b.	12 psf (575 Pa) for inswing single.

c.	8 psf (383 Pa) for inswing pair.

3.	Uniform Load Deflection:  There shall be no deflection of any framing member in excess of L/175

of the span when tested in accordance with ASTM E 330 and AAMA 101 (5.3.4.2) at a differential

static pressure of:

a. 80.0 psf (3830 Pa) for outswing single door, positive and negative.

b. 65.0 psf (3112 Pa) for outswing pair doors, positive and negative.

c. 60.0 psf (2873 Pa) for inswing single door, positive and negative.
			d. 40.0 psf (1915 Pa)  for inswing pair doors, positive and negative.

4.	Uniform Load Structural:  When tested in accordance with ASTM E 330 and AAMA 101(5.3.4.3)

there shall be no permanent deformation of any mainframe, sash, sash member, leaf, or sill in excess

of 0.2% of its span at a differential static pressure of:

a. 120 psf (5745 Pa) for outswing single door, positive and negative.

b. 97.5 psf (4668 Pa) for outswing pair doors, positive and negative.

c. 90.0 psf (4309 Pa) for inswing single door, positive and negative.

d. 60.0 psf (2873 Pa) for outswing pair doors, positive and negative.

In addition, there shall be no permanent damage to fasteners, hardware parts, accessories, or any

other damage, which causes the specimen to be inoperable.

Editor Note: Thermal transmittance and Condensation Resistance values reported below are based on 1” clear high performance insulating glass, 1/4” Clear, 1/2” Air Space, 1/4” Clear (E=0.034 #3), having a center of glass U-factor of 0.29 BTU/hr/SF/°F.

Since system performance is dependent on the specified glass, the NFRC 100 overall system U-factor and NFRC 200 SHGC may be obtained by referencing the AAMA 507 Certificate of Compliance for this product, available at www.ykkap.com.

5.	Thermal Performance:  When tested in accordance with AAMA 1503 and NFRC 102:

a. Condensation Resistance Factor (CRFf): A minimum of 56.

b. Thermal Transmittance U Value: 0.44 BTU/HR/FT²/°F or less.

The Condensation Resistance rating (CR) has been simulated using various glass types and may be obtained by contacting a YKK AP engineering representative at 1-866-955-2732.

6.	Acoustical Performance: When tested in accordance with AAMA 1801, the Sound Transmission

Class (STC) and Outdoor-Indoor Transmission Class (OITC) shall not be less than;

a. 35 STC, 29 OITC; tempered glass.

b. 38 STC, 32 OITC; laminated glass.

7.	Life Cycle Testing: When tested in accordance with AAMA 910, there shall be no damage to fasteners, hardware

parts, or any other damage that would cause the specimen to be inoperable. Resistance to air leakage and water

penetration resistance test results shall not exceed the gateway performance.

8.	Operation/Cycling Performance: Architectural Terrace Doors shall demonstrate compliance with

AAMA 920 at the 25,000 cycle level of operation.

9.	Vertical Loading Resistance: Architectural Terrace Doors shall comply with AAMA 925 for performance class “AW” with a 500 lb. loading.

10.	Operating Force:  Architectural Terrace Doors shall conform to AAMA 101 (5.3.1.2) for both Latch

and Deadbolt “Force to Latch” requirements.

11.	Forced Entry Resistance:  Architectural terrace doors shall be tested in accordance with AAMA

1304.

1.03	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with “Conditions of

the Contract” and Division 1 Submittals Section. Product data, shop drawings, samples, and similar

submittals are defined in “Conditions of the Contract”.

B.	Product Data:  Submit complete product data including installation instructions.

C.  Substitutions: Whenever substitute products are to be considered, supporting technical data, samples,

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

D.	Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors, and hardware.

E.	Samples:  Submit verification samples for colors.  Minimum 2 ½ inch by 3 inch (61mm by 73mm)

samples on actual aluminum substrates indicating full color range expected in installed system.

F.	Quality Assurance / Control Submittals:

1.  	Test Reports:   Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

2.  	Installer Qualification Data:  Submit installer qualification data.

G.	Close-out Submittals:

1.  	Warranty:  Submit executed warranty documents specified herein, endorsed by YKK AP authorized

official and installer.

2.  	Project Record Documents:  Submit project record documents, including operation and maintenance data for installed materials in accordance with Division 1 Project Close-out (Project Record Documents) Section.

1.04	QUALITY ASSURANCE

A.	Qualifications:

1.	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and manufacturer approved installation methods. Obtain Owner's and Architect's acceptance of finish color, and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation: Mock-up may be incorporated into final construction upon Owner's approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer's installation instructions, and manufacturer's warranty requirements.

D. Field Testing:

1.	Field testing, if required by the Architect or Owner, shall be conducted in accordance with AAMA

502-02.

1.05	PRODUCT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings.  Coordinate field measurements and fabrication

schedule with construction progress to 			avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01	MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: 	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

B. Aluminum Architectural Terrace Doors and Frames

1.	AAMA Designation:

Outswing

a.	AW-PG80-ATD for single doors.

b.	AW-PG65-ATD for pair doors.

Inswing

a.	AW-PG60-ATD for single doors.

b.	AW-PG40-ATD for pair doors.

2.	Description: YKK AP Series YTD 350T Thermally Broken Architectural Terrace Doors shall be

extruded aluminum with an overall frame depth of 3-1/2" (88.9mm) or 4-1/2" (114.3mm); Door

Frame members shall be square cut, and notched, factory sealed and assembled, Door Panel

members to be mitered cut, factory sealed, and assembled.

3.	Configuration: The YTD 350 T to be outswing single or pair, or inswing single or pair.

4.	Thermal Barrier: Provide continuous thermal barrier by means of 6/6 nylon polyamide glass fiber

reinforced pressure extruded bars. Systems employing non-structural thermal barriers are not

accepted.

2.02	MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 and 6063-T6 Aluminum Alloys.

B.	Aluminum Sheet:

1.	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050” (1.27 mm) 
			minimum thickness

2.	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080” (1.95 mm)

minimum thickness.

2.03	ACCESSORIES

A.	Manufacturer’s Standard Accessories:

1.	Standard Terrace Door Hardware: Provide hardware units indicated in sizes, number and type

recommended by manufacturer for doors indicated.  Finish exposed parts to match door finish,

unless otherwise indicated.

2.	Hinges: Provide manufacturer’s standard three way adjustable hinges as specified in approved shop

drawings.

3.	Locks: Provide manufacturers standard multi-point locking system that secures the locking stile, top

rail and bottom rail to the frame for enhanced security and performance. Locking system to include

a latch lock and thumb-turn actuated dead bolt for additional security

4.	Fasteners: All fasteners to be AISI 300 series (except for self-drilling which are to be AISI 400

series) stainless steel.

5.	Sealant:  Non-skinning type, AAMA 803.3.

2.04	 FABRICATION

A.	Frame members run through vertically, with notched horizontal members, butted and mechanically

fastened with two fasteners per joint into integral screw splines. Door leaf has mitered frame corners,

butted and mechanically fastened with two fasteners per joint into each corner key for extra strength and

weatherability. Accurately fit and secure joints and corners.

1.	Hardware:  Attach cylinder and handle hardware as instructed in the YKK AP YTD 350 T

installation manual. Attach optional hardware per manufacturer's instructions.

2.05	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE	DESCRIPTION

YS1N 	Clear Anodized Plus®

YH3N	Champagne Anodized Plus®

YB1N	Medium Bronze Anodized Plus®

YB5N 	Dark Bronze Anodized Plus®

YK1N 	Black Anodized Plus®

YW3N	White Anodized Plus®

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with the requirements of AAMA 612-02. Aluminum extrusions shall be produced from quality controlled billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612-02: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of

Anodic Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall

demonstrate the ability of the finish to resist damage from mortar, salt spray, and chemicals

commonly found on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1. 	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors:  Selected by Architect from the following:

a.	Standard coating color charts.

b. 	Custom coating color charts.

c. 	Color Name and Number:

D.	Finishes Testing:

1.  	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place for sixty minutes; lightly wipe off NaOh. Do not clean area further.

2.  	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS / RECOMMENDATIONS

A. 	Compliance:  Comply with manufacturer’s product data, including product technical bulletins,

installation instructions, and product carton instructions.  The latest installation manual is available at

www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify conditions (which have been previously installed under other

sections) are acceptable for product installation in accordance with manufacturer’s instructions.

1. 	Verify location of perimeter fasteners and block-outs are in accordance with shop drawings.

3.03	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04	INSTALLATION

A.	General:  Install manufacturer’s system in accordance with shop drawings, manufacturer’s installation

instructions, and within specified tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials.

2. 	Shim and brace architectural terrace door frame before anchoring to structure.

3.	Completed architectural terrace doors must allow water to be wept to the exterior; Verify weep

holes are open.

4. 	Seal metal to metal door frame joints using sealant recommended by system manufacturer.

3.05	FIELD QUALITY CONTROL

A.	Manufacturer’s Field Services:  Upon request, provide manufacturer’s field service consisting of site visit for inspection of product installation in accordance with manufacturer’s instructions.

B.	Field Test:  Conduct field test to determine watertightness of architectural terrace door. Conduct test in

accordance with AAMA 502-02 at locations selected by Architect.

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust architectural terrace doors for operation in accordance with manufacturer’s

recommendations.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer’s

instructions prior to Owner’s acceptance, and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product’s finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.