PART 1 GENERAL

1.01	SUMMARY

A.	Section Includes:  Glazed Unitized Aluminum Curtain Walls:

1.	YKK AP Series YUW 750 TU Unitized Aluminum Curtain Wall System

B.	Related Sections:

1. 	Sealants:  Refer to Division 7 Joint Treatment Section for sealant requirements.

2. 	Glass and Glazing:  Refer to Division 8 Glass and Glazing Section for glass and glazing

requirements.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum-Framed Glass Doors.

b.	Section 08 41 13 Aluminum-Framed Entrances and Storefronts.

c.	Section 08 51 13 Aluminum Windows.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02	SYSTEM PERFORMANCE DESCRIPTION

A.	Performance Requirements:  Provide aluminum curtain wall systems that comply with performance

requirements indicated, as demonstrated by testing manufacturer’s assemblies in accordance with test

method indicated.

1. 	Wind Loads:  Completed curtain wall system shall withstand wind pressure loads normal to wall

plane indicated:

a. 	Exterior Walls:

1) 	Positive Pressure:

2) 	Negative Pressure:

b. 	Interior Walls (Pressure Acting in Either Direction):

2. 	Deflection:  Maximum allowable deflection in any member when tested in accordance with ASTM E 330 with allowable stress in accordance with AA Specifications for Aluminum Structures.

a. 	For spans less than 13’-6” (4.1m):  L/175.

b. 	For spans greater than 13’-6” (4.1m):  L/240 + 1/4” (6.4mm).

3. 	Thermal Movement:  Provide for thermal movement caused by 180 degrees F. (82.2 degrees C.)

surface temperature, without causing buckling stresses on glass, joint seal failure, undue stress on

structural elements, damaging loads on fasteners, reduction of performance, or detrimental effects.

4. 	Air Infiltration:  Completed curtain wall systems shall have 0.06 CFM/FT² (1.10 m³/h·m²) maximum

allowable infiltration when tested in accordance with ASTM E 283 at differential static pressure of

6.24 PSF (299 Pa).

5.  Water Infiltration:

a.	No uncontrolled water on indoor face of any component when tested in accordance with ASTM E 331 at a static pressure of 25 PSF (1197 Pa).

b.	No uncontrolled water on indoor face of any component when tested in accordance with AAMA

501.1 at a dynamic pressure of 25 PSF (1197 Pa).

6. 	Thermal Performance for YUW 750 XT when tested in accordance with AAMA 1503 and NFRC

102:

a. 	Condensation Resistance Factor (CRFf):  A minimum of 73 for Captured, 72 for SSG.

b. 	Thermal Transmittance U Value:

4 Side Captured: 0.41 BTU/HR/FT²/ºF or less.

4 Side SSG: 0.41 BTU/HR/FT²/ºF or less.

Note:  Performance based on lab testing and will vary by glass type; see actual test reports.

7.	Acoustical Performance: When tested in accordance with ASTM E 1425:

a.	Sound Transmission Class (STC) shall not be less than:

Captured: 32 with 1” Annealed Glass, 36 with 1” Laminated Glass

SSG: 33 with 1”Annealed Glass, 36 with 1” Laminated Glass

b.	Outdoor–Indoor Transmission Class (OITC) shall not be less than:

Captured: 27 with 1” Annealed Glass, 30 with 1” Laminated Glass

SSG: 27 with 1”Annealed Glass, 30 with 1” Laminated Glass

8.	Seismic Movement:  When tested in accordance with AAMA 501.4:

a.	Horizontal Inter-story Differential Lateral Movement Test:  Passed with design displacement of

0.010 times the story height and meets the performance criteria for a Highest Occupancy

Assembly (Group III in NEHRP Provisions).

b.  Vertical Displacement Test: Passed with design displacement of ± 0.75".

c.	Seismic Movement Test:  Passed at 1.5 times design displacement.

Note:  Performance based on lab testing and will vary by glass type; see actual test reports.

1.03	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with “Conditions of

the Contract” and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in “Conditions of the Contract.”

B.	Product Data:  Submit product data for each type curtain wall series specified.

C.	Substitutions:  Whenever substitute products are to be considered, supporting technical data, samples

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

- Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors and textures.

E.	Samples:  Submit verification samples for colors on actual aluminum substrates indicating full color

range expected in installed system.

F.	Quality Assurance / Control Submittals:

1. 	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

2. 	Installer Qualification Data:  Submit installer qualification data.

G. 	Closeout Submittals:

1. 	Warranty:  Submit warranty documents specified herein.

2. 	Project Record Documents:  Submit project record documents for installed materials in accordance

with Division 1 Project Closeout (Project Record Documents) Section.

1.04	QUALITY ASSURANCE

A.	Qualifications:

1. 	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner’s and Architect’s acceptance of finish color,

and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation:  Mock-up may be incorporated into final construction upon Owner’s approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer’s installation instructions, and manufacturer’s warranty requirements.

1.05	PROJECT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings. Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01	MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: 	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

1. 	Curtain Wall System: YKK AP YUW 750 TU Aluminum Unitized Curtain Wall System.

B.	Curtain Wall Framing System:

1. 	Description:  Framing shall be thermally broken. Horizontal and vertical framing members shall have

a nominal face dimension of 2-1/2 inches. Depth as indicated on drawings.

2.02	MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 and 6063-T6 Aluminum Alloys.

B.	Aluminum Sheet:

1. 	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050” (1.27 mm)

minimum thickness.

2. 	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080” (1.95 mm)

minimum thickness.

2.03	ACCESSORIES

A.	Manufacturer’s Standard Accessories:

1. 	Fasteners:  Zinc plated steel concealed fasteners; Hardened aluminum alloys or AISI 300 series

stainless steel exposed fasteners, countersunk, finish to match aluminum color.

2. 	Sealant:  Non-skinning type, AAMA 803.3

3. 	Glazing:  Setting blocks, edge blocks, and spacers in accordance with ASTM C 864, shore durometer

hardness as recommended by manufacturer; Glazing gaskets in accordance with ASTM C 864.

2.04	RELATED MATERIALS (Specified In Other Sections)

A.	Glass:  Refer to Division 8 Glass and Glazing Section for glass materials.

2.05	FABRICATION

A.	Shop Assembly:  Fabricate and assemble units with joints only at intersection of aluminum members with hairline joints;

rigidly secure, and sealed in accordance with manufacturer’s recommendations.

2.06	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish (for exterior covers and adaptors):

CODE	DESCRIPTION

YS1N*	Clear Anodized Plus®

YH3N	Champagne Anodized Plus®

YB1N	Medium Bronze Anodized Plus®

YB5N*	Dark Bronze Anodized Plus®

YK1N*	Black Anodized Plus®

YW3N	White Anodized Plus®

M			Mill Finish (for hidden parts)

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing (for exterior covers and adaptors):  Prepare aluminum surfaces for specified finish;

apply shop finish in accordance with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612: Voluntary					Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1. 	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors:  Selected by Architect from the following:

a.	 Standard coating color charts.

b. 	Custom coating color charts.

c. 	Color Name and Number:

D.	Finishes Testing:

1. 	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh; Do not clean area further.

2. 	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS / RECOMMENDATIONS

A.	Compliance:  Comply with manufacturer’s product data, including product technical bulletins,

installation instructions, and product carton instructions.  The latest Installation Manual can be found

at www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify substrate conditions (which have been previously installed under

other sections) are acceptable for product installation in accordance with manufacturer’s instructions.

3.03	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04	INSTALLATION

A.	General:  Install manufacturer’s system in accordance with shop drawings, and within specified

tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials using

nylon pads or bituminous coating.

2. 	Shim and brace aluminum system before anchoring to structure.

3.  	Verify curtain wall system allows water entering system to be collected in gutters and wept to the

exterior. Verify weep holes are open, and metal joints are sealed in accordance with manufacturers

installation instructions.

4.  	Seal metal to metal curtain wall system joints using sealant recommended by system manufacturer.

3.05	FIELD QUALITY CONTROL

A.	Manufacturer’s Field Services:  Upon request, provide manufacturer’s field service consisting of site

visit for inspection of product installation in accordance with manufacturer’s instructions.

B.	Field Test:  Conduct field test to determine watertightness of curtain wall system. Conduct test in

accordance with AAMA 501.2.

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust operating items as recommended by manufacturer.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer’s

instructions prior to Owner’s acceptance, and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect installed product’s finish surfaces from damage during

construction.

END OF SECTION

This document supersedes all previous versions.