PART 1 GENERAL

1.01	SUMMARY

A.	Section Includes: Aluminum Swing Doors, including:

1. 	YKK AP Model 25FD Flush Panel Swing Entrances.

B.	Related Sections:

1.	Glass and Glazing:  Refer to Division 8 Glass and Glazing Section for glass and glazing

requirements.

2.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum - Framed Glass Doors.

b.	Section 08 51 13 Aluminum Windows.

c.	Section 08 44 13 Glazed Aluminum Curtain Walls.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02	SYSTEM PERFORMANCE DESCRIPTION

A.	Performance Requirements: Provide flush panel swing entrances that comply with performance requirements indicated, as demonstrated by testing manufacturer’s assemblies in accordance with test methods indicated.

1.	Air Infiltration):  Air infiltration shall be tested in accordance with 	ASTM E 283 at static pressure of 1.57 PSF (75 Pa). Infiltration shall not exceed 0.50 CFM/FT² for single door, 1.0 CFM/FT² for pair doors.

2.	Structural Uniform Load Test: +75 / -50 PSF.

3.	Forced Entry Resistance:  Grade 10.

1.03	PROJECT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings. Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.04	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with "Conditions of

the Contract" and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in "Conditions of the Contract."

B.	Product Data:  Submit product data for each entrance series specified.

C.	Substitutions: Whenever substitute products are to be considered, supporting technical data, samples,

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

D.	Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, and finish colors.

E.	Samples:  Submit verification samples for colors. Minimum 2-1/2 inch by 3 inch (61 mm by 73 mm)

samples on actual aluminum substrates indicating full color range expected in installed system.

F.	Quality Assurance / Control Submittals:

1.	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

- Installer Qualification Data:  Submit installer qualification data.

G.	Closeout Submittals:

1.		Warranty:  Submit executed warranty documents specified herein, endorsed by YKK AP authorized official and installer.

2.	Project Record Documents:  Submit project record documents, including operation and maintenance

data for installed materials in accordance with Division 1 Project Closeout (Project Record

Documents) Section.

a.	Maintenance Data:  Maintenance procedures for care and cleaning of entrance systems.

1.05	QUALITY ASSURANCE

A.	Qualifications:

1.	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner's and Architect's acceptance of finish color,

and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation: Mock-up may be incorporated into final construction upon Owner's approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer's installation instructions and manufacturer's warranty requirements.

WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01 MANUFACTURERS

A.	Acceptable Manufacturers:		YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

- Door Frames:  YKK AP VersaJamb® Tubular Aluminum Door Frames.

2. 	Hardware:  Manufacturer's standard, or custom as selected by Architect.

3. 	Flush Panel Swing Doors:  YKK AP Model 25FD Swing Doors.

a.   Description:  1-3/4" (54.0 mm) Door Stile.

4.	Glazing Stops:  Manufacturer's standard snap-in glazing stops with EPDM glazing gaskets to prevent

water infiltration.

5.	Weather-stripping:  Manufacturer's standard pile type in replaceable rabbets for stiles; manufacturer's

standard EPDM bulb type in door frames.

6. 	Hardware:  Manufacturer's standard, or custom as selected by Architect.

7.	Requests for substitution will be considered in accordance with provisions of Section 01 60 00

[01600].

2.02 MATERIALS

A. YKK AP VersaJamb® Tubular Aluminum Door Frame:

1.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 Aluminum Alloy.

a.	Anodized Finish:  AAMA 612-05.

b. 	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080” (1.95) mm) 
minimum thickness.

B. Flush Panel Door:

1.	Minimum Thickness: 1¾ inches (44 mm), 3-ply composite laminate system.

2.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 Aluminum Alloy.

3.	Face Panels:

a.	Exterior and Interior FRP Panels shall be 0.125-inch (3.18mm) exterior grade; fiberglass reinforced polyester panels. UV- inhibitors shall be the maximum amount formulated within the resin.

b.	Exterior and Interior FRP panels shall have a Class "C" rating for flame spread and smoke generation.

c.	Exterior and Interior Aluminum panels shall be minimum 5-ply composite laminated construction to include 
one-piece 0.040-inch (1.02mm) smooth or vertically ribbed embossed pattern 5005-H14 stretcher-leveled aluminum alloy.

d.  Aluminum Panel Finish: Clear and Dark Bronze

e.  FRP Resin Pebble Finish: White, Grey, Dark Bronze and Beige

4.		The door core shall consist of marine grade high compression honeycomb and internal aluminum hardware backup tube.

5.	The hardware backup tube shall be 4.25-inches in width, 1.375-inches in depth with a wall thickness of 0.125-inches.

7.	Internal reinforcing tube shall be mechanically connected top and bottom with galvanized #10 self-tapping screws. 
A 3/8-inch galvanized steel through rod, with stainless steel fiber lock nuts on both ends of the rods, is optional.

6.	Door perimeter shall receive mechanically locked 6063-T5 extruded aluminum alloy bevel edge on the lock stile and a clip mortise square edge on the hinge stile to protect flush door edges and permit field replacement.

7.	Lock stile of door shall have wool pile weather-stripping applied into an extruded aluminum receiver channel in the door edging. This weather stripping to be field replaceable.

C.	Glazing: [Note: Select items below relevant to this project]

1.	Glass shall be 0.25-inch (6.36 mm) tempered. (Standard thickness)

– or select –

1.	Glass shall be 1-inch (25 mm) insulating, tempered.

– or select –

1.	Glass shall be 0.5625-inch (14.29 mm) laminated hurricane glass.

[Note: Retain one of the above to specify glass thickness.]

2.	Stops shall be snap-in, non-removable 6063-T5 extruded aluminum alloy and 0.050-inch (1.25 mm)

thickness.

3.	Seals shall be vinyl inserts.

4.	No fasteners shall be exposed.

D.	Door Louvers:

1.	Blades and Frames: 6063-T5 extruded aluminum alloy, 0.062-inch (1.57 mm) minimum thickness. Louver blades shall be inverted “Y” type.

2.	Insect screens: 14-18 mesh, 0.011-inch (0.28 mm) diameter alclad aluminum, set in 6063-T5 extruded aluminum alloy frame, 0.050-inch (1.25 m) minimum thickness.

3.	Louver shall have a minimum of 50-percent free airflow.

2.03	ACCESSORIES

A.	Manufacturer's Standard Accessories:

1.	Fasteners:  Zinc plated steel concealed fasteners; Hardened aluminum alloys or AISI 300 series

stainless steel exposed fasteners, countersunk, finish to match aluminum color.

2.	Sealant:  Non-skinning type, AAMA 803.3.

3.	Glazing:  Setting blocks, edge blocks, and spacers in accordance with ASTM C 864, shore durometer

hardness as recommended by manufacturer; glazing gaskets in accordance with ASTM C 864.

2.04	RELATED MATERIALS (Specified In Other Sections)

A.	Glass:  Refer to Division 8 Glass and Glazing Section for glass materials.

2.05	FABRICATION

A.	Shop Assembly:  Fabricate and assemble units with joints only at intersection of aluminum members with uniform hairline joints; rigidly secure, and sealed in accordance with manufacturer's recommendations.

1.	Hardware:  Drill and cut to template for hardware. Reinforce frames and door stiles to receive

hardware in accordance with manufacturer's recommendations.

2.06 FINISHES

1.  Door Frames:

a.	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with the requirements of AAMA 612-02. All YKK AP Anodized Plus® finishes available.

b.	High Performance Organic Coating Finish: Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by Solvay Solexis, fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2.	Fiberglass Reinforced Polyester Panels (FRP):

a.	Four standard colors (White, Dark Bronze, Beige, and Gray) with other colors available					 b.	Random pebble texture standard with other textures available.

3.	Aluminum Panels: Electrolytic coating applied in accordance with AAMA 611-98.

a.	Clear anodic coating and dark bronze anodic coating are standard.

b.	Other anodized finishes available.

PART 3 EXECUTION

3.01	MANUFACTURER'S INSTRUCTIONS / RECOMMENDATIONS

A.	Compliance:  Comply with manufacturer's product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation instructions are available

at www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify conditions (which have been previously installed under other

sections) are acceptable for product installation in accordance with manufacturer's instructions.

1.	Verify location of preset anchors, perimeter fasteners, and block-outs are in accordance with shop

drawings.

3.03	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

1.	Aluminum Surface Protection:  Protect aluminum surfaces from contact with lime, mortar, cement,

acids, and other harmful contaminants.

3.04	INSTALLATION

A.	General:  Install manufacturer's system in accordance with shop drawings, and within specified

tolerances.

1.	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials using

nylon pads or bituminous coating.

2.	Shim and brace aluminum system before anchoring to structure.

3.05	FIELD QUALITY CONTROL

A.	Manufacturer’s Field Services:  Upon request, provide manufacturer’s field service consisting of site visit for inspection of product installation in accordance with manufacturer’s instructions.

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust swing doors for operation in accordance with manufacturer's recommendations.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer's

instructions prior to owner's acceptance, and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product's finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.