Editor’s Note:	Luminance® Light Shelf Systems are designed for use with all YKK AP Curtain Wall, Window Wall and other framing systems with a minimum 2" width (sightline).  The section below is intended for insertion into the Guide Specifications for those products.

2.03	ACCESSORIES

A.	Luminance® Light Shelf System – comprised of aluminum support struts attached to vertical mullions (minimum

2” wide) of curtain wall or window wall framing at spans no greater than 84".   Light shelf panels are suspended

between struts.  System depth is up to a nominal 30" plus decorative trims.

B.	Light Shelf Panel:

1.	Composite consisting of marine grade honeycomb core with melamine facers.  Top surface shall be white.

2.	Perimeter channel shall be aluminum extrusions with pivot fasteners to facilitate suspended, drop down

cleaning feature.

3.	Damaged panels can be individually removed and replaced.

C.	Light Shelf Panel interior fascia – panel designed to accept a number of optional decorative trim shapes.

D.	Support struts shall be painted or anodized.

E.	Light shelf panel perimeter and trim shall be painted.

09-3001-01