PART 1 GENERAL

1.01	SUMMARY

A.	Section Includes:  Aluminum Window Wall Systems

1. 	YKK AP Series YHW 60 TU ThermoBond Plus® Aluminum Window Wall System.

B.	Related Sections:

1. 	Sealants:  Refer to Division 7 Joint Treatment Section for sealant requirements.

2. 	Glass and Glazing:  Refer to Division 8 Glass and Glazing Section for glass and glazing requirements.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum-Framed Glass Doors.

b.	Section 08 44 13 Glazed Aluminum Curtain Wall.

c.	Section 08 44 33 Sloped Glazing Assemblies.

d.	Section 08 51 13 Aluminum Windows.

1.02 	SYSTEM PERFORMANCE DESCRIPTION

A.	 Performance Requirements:  All test unit sizes and configurations shall conform to the minimum sizes

in accordance with:  ASTM E 1886 / 1996 or  TAS 201 / 202 / 203, or test protocols for Florida High

Velocity Hurricane Zone (HVHZ).  They shall also comply with the following specific performance

requirements.

- Wind Loads:  Completed window system shall withstand wind pressure loads normal to wall

plane indicated:

a. 	Exterior Walls:

1) 	Positive Pressure:

2) 	Negative Pressure:

b. 	Interior Walls (Pressure Acting in Either Direction):

2. 	Deflection:  Maximum allowable deflection in any member when tested in accordance with

ASTM E 330 with allowable stress in accordance with AA Specifications for Aluminum

Structures.

a. 	For spans less than 13’-6” (4.1m):  L/175 or 3/4” (19.1mm) maximum. .

b. 	For spans greater than 13’-6” (4.1m) but less than 40’-0” (12.2m):  L/175 or L/240 + 1/4”

(6.4mm).

3. 	Thermal Movement:  Provide for thermal movement caused by 180 degrees F. (82.2 degrees C.)

Surface temperature, without causing buckling stresses on glass, joint seal failure, undue stress on

structural elements, damaging loads on fasteners, reduction of performance, or detrimental effects.

4.  	Air Infiltration:  Completed window systems shall have 0.06 CFM/FT2 (1.10 m3/h•m2) maximum allowable infiltration when tested in accordance with ASTM E 283.

5.  	Water Infiltration:  No uncontrolled water on indoor face of any component when tested in accordance with

ASTM E 331 at a static pressure of 15 PSF (719 Pa).

6. 	Thermal Performance for YHW 60TU shall be:

a. 	Condensation Resistance Factor (CRFf):  A minimum of 71 when tested in accordance with

AAMA 1503.1.

b. 	Thermal Transmittance U-Factor:  0.42 BTU/HR/FT²/°F or less in accordance with AAMA

507.

Note:  Thermal performance depends on glass specified. U-Factor shown for system when using a 1-5/16" insulating glass unit - 1/4" clear, 1/2" air space with aluminum spacer, 1/4" clear with a low emissivity coating (e = 0.062) on surface #3.

7.	Acoustical Performance: When tested in accordance with ASTM E 90 and ASTM E 1425, the Sound Transmission

Class (STC) and Outdoor-Indoor Transmission Class (OITC) shall not be less than 39 STC and 33 OITC.

Note:  Performance based on lab testing and will vary by glass type; see actual test reports.

1.03 	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with “Conditions of

the Contract” and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in “Conditions of the Contract.”

B.	Product Data:  Submit product data for each type window series specified.

C.	Substitutions:  Whenever substitute products are to be considered, supporting technical data, samples

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

D.	Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors and textures.

E.	Samples:  Submit verification samples for colors on actual aluminum substrates indicating full color

range expected in installed system.

F.	Quality Assurance / Control Submittals:

1. 	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

G. 	Closeout Submittals:

1. 	Warranty:  Submit warranty documents specified herein.

2. 	Project Record Documents:  Submit project record documents for installed materials in accordance

with Division 1 Project Closeout (Project Record Documents) Section.

1.04 	QUALITY ASSURANCE

A.	Qualifications:

1. 	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner’s and Architect’s acceptance of finish color,

and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation:  Mock-up may be incorporated into final construction upon Owner’s approval..

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer’s installation instructions, and manufacturer’s warranty requirements.

1.05	PROJECT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings. Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01 MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

1. 	Ribbon Window Framing System:  YKK AP YHW 60TU Aluminum Window Wall System.

B.	Window Framing System:

1. 	Description:  Offset flush glazed; jambs and two-piece vertical mullions attached by screw 							spline joinery.  Designed for pre-glazing or inside glaze in the field.                           	    			2. 	Components:  Manufacturer’s standard extruded aluminum mullions, hinged mullions, 90 degree

corner posts, and indicated shapes.

3. 	Thermal Barrier:  Provide YKK AP YWW 50 T MegaTherm® thermal barrier by means of 6/6 nylon

polyamide glass fiber reinforced pressure extruded bars. Systems employing non-structural type

thermal barriers are not acceptable.

2.02 	MATERIALS

A. 	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 Aluminum Alloy.

B.	Aluminum Sheet:

1. 	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050” (1.27 mm)

minimum thickness.

2. 	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080” (1.95 mm)

minimum thickness.

2.03	 ACCESSORIES

A.	Manufacturer’s Standard Accessories:

1.	Fasteners:  Zinc plated steel concealed fasteners; Hardened aluminum alloys or AISI 300 series

stainless steel exposed fasteners.

2. 	Glazing:  Setting blocks, edge blocks, and spacers in accordance with ASTM C 864, shore durometer

hardness as recommended by manufacturer; Glazing gaskets in accordance with ASTM C 864.

3.	0.050 Aluminum Sill Flashing End Dams must have 3 point attachment.

2.04 	RELATED MATERIALS (Specified In Other Sections)

A.	Glass:  Refer to Division 8 Glass and Glazing Section for glass materials.

2.05 	FABRICATION

A.	Shop Assembly:  Fabricate and assemble units with joints only at intersection of aluminum members

with hairline joints; rigidly secure, and sealed in accordance with manufacturer’s recommendations.

2.06 	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE	DESCRIPTION

YS1N*	Clear Anodized Plus®

YH3N	Champagne Anodized Plus®

YB1N	Medium Bronze Anodized Plus®

YB5N*	Dark Bronze Anodized Plus®

YK1N*	Black Anodized Plus®

YW3N	White Anodized Plus®

M			Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1. 	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors:  Selected by Architect from the following:

a.	 Standard coating color charts.

b. 	Custom coating color charts.

c. 	Color Name and Number:

D.	Finishes Testing:

1. 	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh; Do not clean area further.

2. 	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01 	MANUFACTURER'S INSTRUCTIONS / RECOMMENDATIONS

A.	Compliance:  Comply with manufacturer's product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation manual is available at

www.ykkap.com.

3.02 	EXAMINATION

A.	Site Verification of Conditions:  Verify substrate conditions (which have been previously installed under

other sections) are acceptable for product installation in accordance with manufacturer’s instructions.

3.03 	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04 	INSTALLATION

A.	General:  Install manufacturer’s system in accordance with shop drawings, and within specified

tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials using

nylon pads or bituminous coating.

2. 	Shim and brace aluminum system before anchoring to structure.

3. 	Provide sill flashing at exterior window systems. Extend extruded flashing continuous with splice

joints; set in continuous beads of sealant.

4.  Verify window system allows water entering system to be collected in gutters and wept to exterior. 				Verify weep holes are open, and metal joints are sealed in accordance with manufacturers installation

instructions.

5.  Locate expansion mullions where indicated on reviewed shop drawings.

6.  Seal metal to metal window system joints using sealant recommended by system manufacturer.

3.05 	FIELD QUALITY CONTROL

A. 	Manufacturer's Field Services:  Upon request, provide manufacturer's field service consisting of site visit

for inspection of product installation in accordance with manufacturer's instructions.

B.	Field Test:  Conduct field test to determine watertightness of window system. Conduct test in

accordance with AAMA 501.2.

3.06 	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust swing doors for operation in accordance with manufacturer's recommendations.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer's

instructions prior to owner's acceptance, and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product's finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.