PART 1 GENERAL

1.01	SUMMARY

A.	Section Includes:  Fixed & Operable Aluminum Window Systems:

1. 	YKK AP Series YOW 225H Fixed Aluminum Window System.

2. 	YKK AP Series YOW 225H Operable Aluminum Window System.

B.	Related Sections:

1. 	Glazing:  Structural Silicone Sealant.

2.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum-Framed Glass Doors.

b.	Section 08 41 13 Aluminum-Framed Entrances & Storefronts.

c.	Section 08 44 13 Glazed Aluminum Curtain Wall.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02	SYSTEM PERFORMANCE DESCRIPTION

A.	All test unit sizes and configurations shall conform to the minimum sizes in accordance with

AAMA/WDMA/CSA 101/I.S.A440-11 with a performance class of AW, performance grade 65 (Operable), 100 (Fixed). Windows shall also meet all requirements of South Florida Building Code

Protocols TAS 201, TAS 202, and TAS 203 and comply with the following specific performance

requirements indicated.

1.	Air Infiltration:  When tested in accordance with ASTM E 283 and TAS 202 at differential static

pressure of 6.24 PSF (299 Pa), completed window systems shall have maximum allowable

infiltration of 0.30 CFM/FT2 (1.85 m3/h•m2).

2. 	Water Infiltration:  No uncontrolled water on indoor face of any component when tested in

accordance with ASTM E 331 and TAS 202 at a static pressure of 12 PSF (575 Pa)  operable, 15 PSF

(718 Pa) fixed.

3.	Static Load:  There shall be no damage to fasteners, hardware, accessories, or any other damage that

would render the window inoperable when tested in accordance with ASTM E 330 and TAS 202 at a

differential static pressure as follows;

a.	Operable windows: Large & Small Missile: ±65 psf

b.	Fixed windows:

Large Missile +70/-90 psf

Small Missile ± 100 psf.

4.	Forced Entry Resistance:  Windows shall meet all test requirements of AAMA 1302.5 and TAS 202.

5.	Large & Small Missile Impact:  Windows shall successfully pass the test requirements of both

ASTM E 1886/E 1996 and South Florida Protocols TAS 201.

6.  Cyclic Load:  Windows shall successfully pass the test requirements of both ASTM E 1886/E 1996

and South Florida Protocols TAS 203.

7.	Acoustical Performance: When tested in accordance with ASTM E 90 and ASTM E 1332, the Sound

Transmission Class (STC), and Outdoor–Indoor Transmission Class (OITC) shall not be less than 36

STC and 26 OITC.

8.	Life Cycle Testing: When tested in accordance with AAMA 910, there shall be no damage to

fasteners, hardware parts, or any other damage that would cause the specimen to be inoperable.

Resistance to air leakage and water penetration resistance test results shall not exceed the gateway

performance.

Note:  Performance based on lab testing and will vary by configuration and glass type; contact YKK AP engineering for job specific analysis at higher performance levels.

1.03	SUBMITTALS

A. General:  Prepare, review, approve, and submit specified submittals in accordance with “Conditions of

the Contract” and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in “Conditions of the Contract.”

B. 	Product Data:  Submit product data for each type window series specified.

C.	Substitutions:  Whenever substitute products are to be considered, supporting technical data, samples,

and test reports

must be submitted ten (10) working days prior to bid date in order to make a valid comparison.

- Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors and textures.

- Samples:  Submit verification samples for colors on actual aluminum substrates indicating full color

range expected in installed system.

F. 	Quality Assurance / Control Submittals:

1. 	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

G. Closeout Submittals:

1. 	Warranty:  Submit warranty documents specified herein.

2. 	Project Record Documents:  Submit project record documents for installed materials in accordance

with Division 1 Project Closeout (Project Record Documents) Section.

1.04	QUALITY ASSURANCE

A.	Qualifications:

1. 	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and manufacturer approved installation methods. Obtain Owner’s and Architect’s acceptance of finish color, and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation:  Mock-up may be incorporated into final construction upon Owner’s approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer’s installation instructions, and manufacturer’s warranty requirements.

1.05	PROJECT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication,

show recorded measurements on shop drawings. Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01	MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: 	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (404) 838-6001

1. 	Operable Window System:  YKK AP YOW 225H Aluminum Window System.

2. 	Fixed Window System:  YKK AP YOW 225H Aluminum Window System

B.	Window Framing System:

1. 	AAMA Designation:  AW-65 (Operable), AW-100 (Fixed).

2.	Description:  The windows shall be extruded aluminum; 2-1/4" frame depth;

Vents shall be flush with frame and have mitered corner construction; Factory-assembled.

- Configuration:  The windows shall be (select one or more) Fixed, Casement Outswing, or Project Out.

4.	Glazing (Contact YKK AP for approved glass types):

a.	Large Missile Impact:  Exterior glazing tape with silicone cap bead; 9/16" monolithic units; Interior silicone compatible setting blocks & structural silicone sealant; Aluminum interior glazing beads; Factory or bench glazed.

b.	Small Missile Impact:  Exterior glazing tape with silicone cap bead; 9/16" monolithic units;

Interior EPDM wedge gaskets; Aluminum interior glazing beads; Factory or bench glazed.

2.02	MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 and 6063-T6 Aluminum Alloys.

B.	Aluminum Sheet:

1. 	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050” (1.27 mm)

minimum thickness.

2. 	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080” (1.95 mm)

minimum thickness.

2.03	ACCESSORIES

A.	Manufacturer’s Standard Accessories:

1.	Hardware:  Standard concealed stainless steel 4 bar hinges for casement outswing and projected

vents, white bronze cam handles and strikes; Optional white bronze roto-operators for casement

outswing vents, exposed white bronze butt hinges for egress casement outswing vents,

aluminum/white bronze push bars for project out vents, white bronze multi-locks in lieu of cam

handles, stainless steel limit stop device.

2. 	Fasteners:  All fasteners to be AISI 300 series (except for self-drilling which are to to be AISI 400 series) stainless steel.

3. 	Sealant:  Non-skinning type, AAMA 803.3

4. 	Glazing:  Setting blocks, edge blocks, and spacers in accordance with ASTM C 864, shore durometer

hardness as recommended by manufacturer; Glazing gaskets in accordance with ASTM C 864.

5.	Glazing Adhesive:  Structural silicone sealant.

2.04	RELATED MATERIALS (Specified In Other Sections)

A.	Glass:  Contact YKK AP for approved glass types.

2.05	FABRICATION

A.	Shop Assembly:  Fabricate and assemble units with joints only at intersection of aluminum members

with hairline joints; rigidly secure, and sealed in accordance with manufacturer’s recommendations.

B.	Fabrication Tolerance:

1. 	Material Cuts: Square to 1/32 inch (0.8 mm) off square, over largest dimension; proportionate

amount of 1/32 inch (0.8 mm) on the two dimensions.

2. 	Maximum Offset: 1/64 inch (0.4 mm) in alignment between two consecutive members in line, end to

end.

3. 	Maximum Offset: 1/64 inch (0.4 mm) between framing members at glazing pocket corners.

4. 	Joints (Between adjacent members in same assembly): Hairline and square to adjacent member.

5. 	Variation (In squaring diagonals for doors and fabricated assemblies): 1/16 inch (1.6 mm).

6. 	Flatness (For doors and fabricated assemblies): +/- 1/16 inch (1.8 mm) off neutral plane.

2.06	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE		DESCRIPTION

YS1N*		Clear Anodized Plus®

YH3N		Champagne Anodized Plus®

YB1N		Medium Bronze Anodized Plus®

YB5N*		Dark Bronze Anodized Plus®

YK1N*		Black Anodized Plus®

YW3N		White Anodized Plus®

M				Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612-02. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612-02: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1. 	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer-based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors:  Selected by Architect from the following:

a.	Standard coating color charts.

b. 	Custom coating color charts.

c. 	Color Name and Number:

D.	Finishes Testing:

1. 	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh; Do not clean area further.

2. 	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS / RECOMMENDATIONS

A.	Compliance:  Comply with manufacturer’s product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation instructions are available

at www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify substrate conditions (which have been previously installed under

other sections) are acceptable for product installation in accordance with manufacturer’s instructions.

3.03	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04	INSTALLATION

A.	General:  Install manufacturer’s system in accordance with shop drawings, and within specified

tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials using

nylon pads or bituminous coating.

2. 	Shim and brace aluminum system before anchoring to structure.

3. 	Verify window system allows water entering system to be collected in gutters and wept to the

exterior. Verify weep holes are open, and metal joints are sealed in accordance with manufacturers

installation instructions.

4. 	Seal metal to metal window system joints using sealant recommended by system manufacturer.

3.05	FIELD QUALITY CONTROL

A.	Manufacturer’s Field Services:  Upon request, provide manufacturer’s field service consisting of site

visit for inspection of product installation in accordance with manufacturer’s instructions.

B.	Field Test:  Conduct field test to determine watertightness of window system. Conduct test in

accordance with AAMA 502-02 at locations selected by Architect.

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust operating items as recommended by manufacturer.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer’s

instructions prior to Owner’s acceptance and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect installed product’s finish surfaces from damage during

construction.

END OF SECTION

This document supersedes all previous versions.