PART 1 GENERAL

Furnish all necessary materials, labor, and equipment for the complete installation of the aluminum windows as shown on the drawings and specified herein.

1.01 SUMMARY

A.	Section includes: Aluminum Horizontal Sliding Windows, including:

1. 	YKK AP Series YSW 400 T Sliding Windows (optional with screens).

B.	Related Sections:

1. 	Sealants: Refer to Division 7 Joint Treatment Section.

2. 	Glass and Glazing:  Refer to Division 8 Glass and Glazing Section for glass and glazing

requirements.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum - Framed Glass Doors.

b.	Section 08 41 13 Aluminum - Framed Entrances and Storefronts.

c.	Section 08 44 13 Glazed Aluminum Curtain Walls.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02 SYSTEM PERFORMANCE

A.	All test unit sizes and configurations shall conform to the minimum sizes in accordance with AAMA/WDMA/CSA/I.S.A 440-05, with a performance class of HS-AW, performance grade 40. Windows shall also comply with the following specific performance requirements indicated.

1.	Air Infiltration:  Completed window systems shall have 0.30 CFM/FT² maximum allowable

infiltration when tested in accordance with ASTM E 283 at a differential static pressure of 6.24 psf

(299 Pa).

2.	Water Infiltration:  There shall be no uncontrolled water leakage when tested in accordance with

ASTM E 331 at a static pressure of 9 psf  (431 Pa).

3.	Uniform Load Structural:  There shall be no damage to fasteners, hardware, accessories, or any other

damage that would render the window inoperable when tested in accordance with ASTM E 330 at a

differential static pressure of 60.0 psf (2870 Pa) positive and negative.

4.	Forced Entry Resistance:  Windows shall be tested in accordance with ASTM F 588 and meet the

requirements of performance level 10.

5.	Deglazing:  Windows shall meet all requirements when tested in accordance with ASTM E 987.

6.  Thermal Performance:  When tested in accordance with AAMA 1503.1-88:

a. 	Condensation Resistance Factor (CRF):  A minimum of 55.

b. 	Thermal Transmittance U Value:  0.50 BTU/HR/FT²/°F or less.

7.	Acoustical Performance: When tested in accordance with ASTM E 90 and ASTM E 1332, the Sound

Transmission Class (STC), and Outdoor–Indoor Transmission Class (OITC) shall not be less than 33

STC and 28 OITC.

8.	Life Cycle:  There shall be no damage to fasteners, hardware, accessories, or any other damage that

would render the window inoperable, and air and water infiltration tests shall not exceed the primary

performance specified herein when tested in accordance with AAMA 910.

Note:  Performance based on lab testing and will vary by configuration and glass type; contact YKK AP engineering for job specific analysis at higher performance levels.

1.03 SUBMITTALS

A. General:  Prepare, review, approve, and submit specified submittals in accordance with “Conditions of

the Contract” and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in “Conditions of the Contract.”

B. 	Product Data:  Submit product data for each type window series specified.

C.	Substitutions:  Whenever substitute products are to be considered, supporting technical data, samples,

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

- Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors and textures.

E. 	Samples:  Submit verification samples for colors on actual aluminum substrates indicating full color

range expected in installed system.

F. 	Quality Assurance / Control Submittals:

1. 	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

1.04 QUALITY ASSURANCE

A.	Qualifications:

1.	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner's and Architect's acceptance of finish color,

and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation: Mock-up may be incorporated into final construction upon Owner's approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer's installation instructions, and manufacturer's warranty requirements.

1.05 PRODUCT CONDITIONS/SITE CONDITIONS

A.	Field Measurements:  Verify measurements/openings by field measurements before fabrication; show

recorded measurements on shop drawings. Coordinate field measurements and fabrication schedule with

construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01	MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers:	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (404) 838-6001

1.	Sliding Window:  YKK AP YSW 400 T Series Horizontal Sliding Windows.

B.	Window Framing System:

1.	AAMA Designation:  HS-AW 40.

2. 	Description:  Overall frame depth to be 4”. Fixed and sliding panels shall have overlapped joints,

sealed, and dual screw spline joinery for extra strength and weatherability. Sliding panels shall not be

removable when in a locked position. Windows including sliding as well as fixed panels, and frames

shall be factory fabricated.

3.	Thermal barrier shall consist of 6/6 nylon polyamide glass fiber reinforced pressure extruded bars.

Systems employing non-structural thermal barriers are not acceptable.

4.	Configuration:  The sliding windows shall be XO, OX, XOX, or OXO.

5.	Glazing:  Each panel shall be channel glazed using a marine type wrap around EPDM glazing gasket;

1/4” monolithic or 1” insulating units; Factory or bench glazed. Glazing thickness as specified in

Division 8 glass and glazing section.

2.02 MATERIALS

A.	Extrusions: Aluminum Alloy  6063-T5 Temper (ASTM B 221).

B.	Aluminum Sheet:

1.		Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H34 Aluminum Alloy, 0.050” (1.27 mm) minimum thickness.

2. 	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H3 Aluminum Alloy, 0.080” (1.95) mm)

minimum thickness.

2.03 ACCESSORIES

A.	Manufacturer’s Standard Accessories:

1. 	Hardware:  Standard adjustable acetal roller in stainless steel housing, cast aluminum cam-type

sweep lock, and stainless-steel keeper.

2.  	Fasteners: All fasteners to be AISI 300 series (except for self-drilling which are to be AISI 400

series) stainless steel.

3.  	Sealant:  Non-skinning type, AAMA 803.3.

4.  	Glazing:  Marine type wrap around gaskets. EPDM in accordance with ASTM C864. Color Black.

2.04 FABRICATION

A.	Fabricate frame using two screws and adhesive backed gaskets at corner joints. Fixed and sliding panels

shall have over lapped joints, sealed, and dual screw/spline joinery for extra strength and weatherability.

Sliding panels shall not be removable when in a locked position.

1.  	Hardware:  Drill and cut to template for hardware. Reinforce frames and door stiles to receive

hardware in accordance with manufacturer's recommendations.

B.	Fabrication Tolerances:

1.  	Material Cuts:  Square to 1/32 inch (0.8 mm) off square, maximum, over largest dimension;

proportionate amount of 1/32 inch (0.8 mm) on other two dimensions.

2.  	Joints (Between adjacent members in same assembly):  Hairline and square to adjacent member.

3.  	Variation (In squaring diagonals for doors and fabricated assemblies):  1/16 inch (1.6 mm).

4.  	Flatness:  +/- 1/16 inch (1.6 mm) off neutral plane.

2.05 FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE		DESCRIPTION

YS1N*		Clear Anodized Plus®

YH3N		Champagne Anodized Plus®

YB1N		Medium Bronze Anodized Plus®

YB5N*		Dark Bronze Anodized Plus®

YK1N*		Black Anodized Plus®

YW3N		White Anodized Plus®

M				Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612-02. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612-02: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1.  	Fluoropolymer Type: Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2.  	Colors:  Selected by Architect from the following:

a.  	Standard coating color charts.

b.  Custom coating color charts.

c.  	Color Name and Number:

D.	Finishes Testing:

1.  	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh. Do not clean area further.

2.  	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01 MANUFACTURER'S INSTRUCTIONS / RECOMMENDATIONS

A. 	Compliance:  Comply with manufacturer's product data, including product technical bulletins,				installation instructions, and product carton instructions. The latest installation manual is available at

www.ykkap.com.

3.02 EXAMINATION

A.	Site Verification of Conditions:  Verify conditions (which have been previously installed under other

sections) are acceptable for product installation in accordance with manufacturer's instructions.

1. 	Verify location of perimeter fasteners and block-outs are in accordance with shop drawings.

3.03 PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04 INSTALLATION

A.	General:  Install manufacturer’s system in accordance with shop drawings, and within specified

tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials.

2. 	Shim and brace aluminum system before anchoring to structure.

3.	Completed windows must allow water to be wept to the exterior; Verify weep holes are open.

4. 	Seal metal to metal window system joints using sealant recommended by system manufacturer.

3.05 FIELD QUALITY CONTROL

A.	Manufacturer’s Field Services:  Upon request, provide manufacturer’s field service consisting of site

visit for inspection of product installation in accordance with manufacturer’s instructions.

B.	Field Test:  Conduct field test to determine watertightness of window system. Conduct test in

accordance with AAMA 501.2-03.

3.06 ADJUSTING AND CLEANING

A.	Adjusting:  Adjust sliding windows for operation in accordance with manufacturer's recommendations.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer's

instructions prior to Owner's acceptance and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product's finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.