PART 1 GENERAL

1.01	SUMMARY

A.	Section Includes:  Furnish and install YKK AP Architectural Aluminum Windows, complete with hardware and accessories as shown on shop drawings and specified in this section.

1.	YKK AP Series YVS 410 TUH Aluminum ThermaBond Plus® Side-Load Single Hung Windows.

B.	Related Sections:

1. 	Sealants:  Structural silicone sealant.

2. 	Glass and Glazing:

a.	All units shall be factory glazed.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum-Framed Glass Doors.

b.	Section 08 41 13 Aluminum-Framed Entrances & Storefronts.

c.	Section 08 44 13 Glazed Aluminum Curtain Wall.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02	TEST AND PERFORMANCE REQUIREMENTS

A.	Performance Requirements:  Windows shall conform to all AAMA/NAFS 101/I.S.2/NAFS-02

requirements for the window type, meet all requirements of South Florida Building Code Protocols TAS

201, TAS 202, and TAS 203 and comply with the following specific performance requirements

indicated.

1.	Air Infiltration:  Completed window systems shall have 0.30 (operable) CFM/FT² (5.5m³/h•m²) maximum allowable infiltration when tested in accordance with ASTM E 283 and TAS 202 at a differential static pressure of 6.24 psf (299 Pa).

2.	Water Infiltration:  With the sashes closed and locked there shall be no uncontrolled water leakage

when tested in accordance with ASTM E 331 and TAS 202 at a static pressure of 12 PSF (575 Pa).

3.	Static Load:  There shall be no damage to fasteners, hardware, accessories, or any other damage that

would render the window inoperable when tested in accordance with ASTM E 330 and TAS 202 at a

differential static pressure of 80.0 psf positive and 120.0 psf negative.

4.	Large & Small Missile Impact:  There shall be no signs of penetration, rupture, or opening after the

impact test when tested in accordance with ASTM E 1886/1996 and TAS 201.

5.  Cyclic Load:  Test to be done upon completion of missile impact test. There shall be no damage to

fasteners, hardware, accessories, or any other damage that would render the window inoperable when

tested in accordance with ASTM E 1886/1996 and TAS 203.

6.	Forced Entry Resistance:  Windows shall be tested in accordance with ASTM F 588 and TAS 202

and meet the requirements of performance level 10.

7.	Thermal Performance: When tested in accordance with AAMA 1503 and NFRC 102:

a. 	Condensation Resistance Factor (CRF):  A minimum of 54.

b. 	Thermal Transmittance U Value:  0.49 BTU/HR/FT²/ºF or less.

8.	Acoustical Performance: Acoustical Performance: When tested in accordance with ASTM E 90 and ASTM E 1332, the Sound Transmission Class (STC), and Outdoor–Indoor Transmission Class (OITC) shall not be less than 36 STC, and 22 OITC.

- Life Cycle Testing: When tested in accordance with AAMA 910, there shall be no damage to

fasteners, hardware parts, or any other damage that would cause the specimen to be inoperable.

Resistance to air leakage and water penetration resistance test results shall not exceed the gateway

performance.

Note:  Performance based on lab testing and will vary by configuration and glass type; contact YKK AP engineering for job specific analysis at higher performance levels.

1.03	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with “Conditions of

the Contract” and Division 1 Submittals Sections. Product data, shop drawings, samples, and similar

submittals are defined in “Conditions of the Contract.”

B.	Quality Assurance/Control Submittals:

1. 	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

C.	Substitutions:  Whenever substitute products are to be considered, supporting technical data, samples

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

1.04	QUALITY ASSURANCE

A.	Qualifications:

1. 	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2. 	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

the construction process.

B.	Mock-Ups (Field Constructed):  Install at project site a job mock-up using acceptable products and

manufacturer approved installation methods. Obtain Owner’s and Architect’s acceptance of finish color, and workmanship standard.

1. 	Mock-Up Size:

2. 	Maintenance:  Maintain mock-up during construction for workmanship comparison; remove and

legally dispose of mock-up when no longer required.

3. 	Incorporation:  Mock-up may be incorporated into final construction upon Owner’s approval.

C.	Pre-Installation Meetings:  Conduct pre-installation meeting to verify project requirements, substrate

conditions, manufacturer’s installation instructions, and manufacturer’s warranty requirements.

1.05	PROJECT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings. Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

2.01	MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: 	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (404) 838-6001

1. 	Operable Windows:  YKK AP YVS 410 TUH ThermaBond Plus® Impact Resistant Hung (Side-

Load) Windows.

B.	Windows:

1. 	AAMA Designation:  AW-65 (Single Hung).

2.	Description:  The windows shall be extruded aluminum; 4" frame depth; Horizontal frame members

run through notched vertical members, butted and mechanically fastened with two stainless steel

screws per joint; Factory assembled.

3.	Configuration:  The windows shall be Single Hung.

4.	Glazing (Contact YKK AP for approved glass types):

a.	Large Missile Impact: Exterior closed cell EPDM sponge glazing tape with silicone cap bead;

1-1/16" (overall) insulating units; Interior EPDM (silicone compatible) spacer with structural

silicone sealant; Removable, extruded aluminum interior glazing bead; Factory glazed.

b.	Small Missile Impact: Exterior closed cell EPDM sponge glazing tape with silicone cap bead;

1-1/16" (overall) insulating units; Interior EPDM glazing gasket; Removable, extruded aluminum

interior glazing bead; Factory glazed.

5.	Thermal Barrier:  Provide continuous thermal barrier by means of a poured and debridged pocket

consisting of a two-part, chemically curing high density polyurethane which is bonded to the

aluminum by YKK ThermaBond Plus®. Systems employing nonstructural type thermal barriers are

not acceptable.

2.02	MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 Aluminum Alloy.

1.	All members shall have minimum wall thickness sufficient to meet the specified structural

requirements.

2.03	ACCESSORIES

A.	Manufacturer’s Standard Accessories:

1.	Hardware:  Extruded aluminum spring catch head and/or sill locks. Optional stainless-steel sweep

lock and keeper.

2. 	Fasteners:  All fasteners shall be AISI 300 series (except for self-drilling, which are to be series 400)

stainless steel.

3.	Balances:  Select balance required for performance class in accordance with 101/I.S.2/NAFS-02 and

AAMA 902.

a)	Class 2 Block & Tackle balances shall be of appropriate size and capacity to hold sash in position

and meet minimum .70 Manually Applied Force (MAF) ratio.

OR

a)	Class 5 High Performance balances shall be of appropriate size and capacity to hold sash in

position and meet minimum .30 Manually Applied Force (MAF) ratio.

b)	Balances shall be attached to a locking carrier mechanism that slides on extruded rails in the jamb

channels.

4.	Weather-stripping:  All weather-stripping shall be Fin-Seal or equivalent.

5. 	Glazing Materials:  Setting blocks, edge blocks, and spacers in accordance with ASTM C 864, shore

durometer hardness as recommended by manufacturer; glazing gaskets in accordance with ASTM C

864.

6.  Glazing Adhesive:  Structural silicone sealant.

2.04	RELATED MATERIALS (Specified In Other Sections)

A.	Glass:  All windows shall be factory glazed in accordance with manufacturer’s standards.

1.	Insulated glass type and thickness shall be in accordance with manufacturer’s recommendations for

design pressure.

2.05	FABRICATION

A.	Frame:

1.	Horizontal frame members run through notched vertical members, butted and mechanically fastened

with two screws per joint into integral screw splines; Meeting rail notched at each end, butted and

mechanically fastened with two screws per end into integral screw splines.

2.	All framing joints shall be sealed with quality grade sealant meeting AAMA 803.3 to ensure       watertight joint.

B.	Sash:

1.	Sashes shall have notched horizontal members, butted and mechanically fastened to vertical

members with two screws per joint into integral screw splines.

C.	Exterior Panning & Trim:

1.	Exterior panning & trim shall be extruded aluminum of profile and dimensions as detailed on				approved shop drawings.

2.	All joints shall be sealed with quality grade sealant meeting AAMA 803.3 to ensure watertight joint.

D.	Mullions:

1.	Mullions shall be of extruded aluminum of profile and dimensions as detailed on approved shop

drawings.

2.	Mullions must provide adequate structural properties to resist wind pressure as specified herein.

2.06	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE		DESCRIPTION

YS1N*		Clear Anodized Plus®

YH3N		Champagne Anodized Plus®

YB1N		Medium Bronze Anodized Plus®

YB5N*		Dark Bronze Anodized Plus®

YK1N*		Black Anodized Plus®

YW3N		White Anodized Plus®

M				Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612-02. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612-02: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1. 	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer-based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2. 	Colors:  Selected by Architect from the following:

a.	 Standard coating color charts.

b. 	Custom coating color charts.

c. 	Color Name and Number:

D.	Finishes Testing:

1. 	Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh; Do not clean area further.

2.	Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS/RECOMMENDATIONS

A.	Compliance:  Comply with manufacturer’s product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation instructions are available

at www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify substrate conditions (which have been previously installed under

other sections) are acceptable for product installation in accordance with manufacturer’s instructions.

3.03	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

3.04	INSTALLATION

A.	General:  Install manufacturer’s system in accordance with shop drawings, and within specified

tolerances.

1. 	Protect aluminum members in contact with masonry, steel, concrete, or dissimilar materials.

2. 	Shim and brace aluminum system before anchoring to structure.

3.	Completed windows must allow water to be wept to the exterior; Verify weep holes are open and

weep caps are installed correctly.

4. 	Seal metal to metal window system joints using sealant recommended by system manufacturer.

3.05	FIELD QUALITY CONTROL

A.	Manufacturer’s Field Services:  Upon request, provide manufacturer’s field service consisting of site

visit for inspection of product installation in accordance with manufacturer’s instructions.

B.	Field Test:  Conduct field test to determine watertightness of window system. Conduct test in

accordance with AAMA 502-02 at locations selected by Architect..

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust operating items as recommended by manufacturer.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer’s

instructions prior to Owner’s acceptance and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect installed product’s finish surfaces from damage during

construction.

END OF SECTION

This document supersedes all previous versions.