PART 1 GENERAL

Furnish all necessary materials, labor, and equipment for the complete installation of the aluminum

storefront framing system as shown on the drawings and specified herein.

1.01 	SUMMARY

A.	Section includes:  Aluminum Storefront Systems

1. 	YKK AP Series YHS 50 TU Impact Resistant and Blast Mitigating Storefront System (Insulating

Glazing)

B.	Related Sections:

1.	Glass:  Contact YKK AP for approved glass types.

2. 	Glazing:  Dow Corning® 995 Structural Silicone Sealant.

3.	Single Source Requirement:  All products listed below shall be by the same manufacturer.

a.	Section 08 32 13 Sliding Aluminum - Framed Glass Doors.

b.	Section 08 51 13 Aluminum Windows.

c.	Section 08 44 13 Glazed Aluminum Curtain Walls.

d.	Section 08 44 33 Sloped Glazing Assemblies.

1.02	SYSTEM DESCRIPTION

A.	All test unit sizes and configurations shall conform to the minimum sizes in accordance with; 	Florida

High Velocity Hurricane Zone (HVHZ) Protocols, ASTM E 1886, ASTM E 1996, and/or meet all

requirements of TAS 201, TAS 202, and TAS 203. They shall also comply with the following specific

performance requirements indicated.

1. 	Air Infiltration:  Completed storefront systems shall have 0.06 CFM/FT² (1.10 m³/h•m²) maximum

allowable infiltration when tested in accordance with ASTM E 283 at differential static pressure of

6.24 psf (299 Pa).

2. 	Water Infiltration: No uncontrolled water when tested in accordance with ASTM E 331 at test

pressure differential of: 12 PSF (575 Pa) (or when required, field tested in accordance with AAMA

503).  Fastener Heads must be seated and sealed against Sill Flashing on any fasteners that penetrate

through the Sill Flashing.

3.  Wind Loads:  Completed storefront system shall withstand wind pressure loads normal to wall plane

indicated:

a. 	Exterior Walls:

1)  Positive Pressure:

2)  Negative Pressure:

4. 	Deflection:  Maximum allowable deflection in any member when tested in accordance with ASTM E

330 with	allowable stress in accordance with AAMA Specifications for Aluminum Structures.

a. 	For spans less than 13'-6" (4.1m):  L/175 or 3/4" (19.1mm) maximum.

b. 	For spans greater than 13'-6" (4.1m) but less than 40'-0" (12.2m):  L/175 or L/240 + 1/4"

(6.4mm).

5.	Thermal Movement:  Provide for thermal movement caused by 180 degrees F. (82.2 degrees C.)

surface temperature, without causing buckling stresses on glass, joint seal failure, undue stress on

structural elements, damaging loads on fasteners, reduction of performance, or detrimental effects.

6.	Thermal Performance:  When tested in accordance with AAMA 1503, and AAMA 507,

a. 	Condensation Resistance Factor (CRF):  A minimum of 59.

b. 	Thermal Transmittance U-Factor:  0.45 BTU/HR/FT²/°F or less in accordance with AAMA 507.

Note:  Thermal performance depends on glass specified. U-Factor shown for system when using a

1-5/16" insulating glass unit - 1/4" heat strengthened as required with 0.034 low emmissivity coating on

surface #2, 1/2" air space with aluminum spacer, 1/4" heat strengthened / 0.090 PVB interlayer / 1/4"

heat strengthened. Size: 2000mm x 2000mm (78.7 inches x 78.7 inches).

7	Acoustical Performance: When tested in accordance with ASTM E 1425:

a.	Sound Transmission Class (STC) shall not be less than 39

b.	Outdoor–Indoor Transmission Class (OITC) shall not be less than 33

1.03	SUBMITTALS

A.	General:  Prepare, review, approve, and submit specified submittals in accordance with "Conditions of

the Contract" and Division 1 Submittals Section. Product data, shop drawings, samples, and similar

submittals are defined in 				"Conditions of the Contract".

B.	Product Data:  Submit product data for each type storefront series specified.

C.	Substitutions:  Whenever substitute products are to be considered, supporting technical data, samples

and test reports must be submitted ten (10) working days prior to bid date in order to make a valid

comparison.

D.	Shop Drawings:  Submit shop drawings showing layout, profiles, and product components, including

anchorage, accessories, finish colors, and textures.

E.	Samples:  Submit verification samples for colors on actual aluminum substrates indicating full color

range expected in installed system.

F.	Quality Assurance / Control Submittals:

1.	Test Reports:  Submit certified test reports showing compliance with specified performance

characteristics and physical properties.

2.	Installer Qualification Data:  Submit installer qualification data.

G.	Close-out Submittals:

1.	Warranty:  Submit warranty documents specified herein.

2.	Project Record Documents:  Submit project record documents for installed materials in accordance

with Division 1 Project Close-out (Project Record Documents) Section.

1.04 	QUALITY ASSURANCE

A.	Qualifications:

1.	Installer Qualifications:  Installer experienced (as determined by contractor) to perform work of this

section who has specialized in the installation of work similar to that required for this project. If

requested by Owner, submit reference list of completed projects.

2.	Manufacturer Qualifications:  Manufacturer capable of providing field service representation during

construction process.

1.05	PRODUCT CONDITIONS / SITE CONDITIONS

A.	Field Measurements:  Verify actual measurements/openings by field measurements before fabrication;

show recorded measurements on shop drawings.  Coordinate field measurements, fabrication schedule

with construction progress to avoid construction delays.

1.06	WARRANTY

A.	Project Warranty:  Refer to "Conditions of the Contract" for project warranty provisions.

B.	Manufacturer's Warranty:  Submit, for Owner's acceptance, manufacturer's standard warranty document

executed by an authorized company official.

1.	Warranty Period:  Manufacturer’s one (1) year standard warranty commencing on the substantial date of completion for the project provided that the warranty, in no event, shall start later than six (6) months from the date of shipment by YKK AP America Inc.

EDITOR NOTE:  Longer warranty periods are available at additional cost.

PART 2 PRODUCTS

It is the intent of this specification to have a single source responsibility for the supply of the aluminum doors and framing systems on this project. Any deviation from the acceptable manufacturers listed below must be approved in writing by the architect at least ten (10) days prior to bid date.

2.01 MANUFACTURERS (Acceptable Manufacturers/Products)

A.	Acceptable Manufacturers: 	YKK AP America Inc.

101 Marietta Street NW, Suite 2100

Atlanta, GA 30303

Telephone: (678) 838-6000; Fax: (678) 838-6001

1.	Storefront System:  YKK AP YHS 50 TU Impact Resistant and Blast Mitigating Storefront System.

B.	Storefront Framing Systems:

1.	Description:  Center set, exterior flush glazed; jambs and vertical mullions continuous; head, sill,

intermediate horizontal attached by screw spline joinery.  Continuous and wept sill flashing.

2.  Components:  Manufacturer's standard extruded aluminum mullions, entrance doors, framing, and

indicated shapes, perimeter anchor fillers and steel reinforcing as required.

3.	Glazing:  Manufacturer's standard glazing stops with EPDM glazing gaskets to prevent water

infiltration at the exterior and Dow Corning® 995 Structural Silicone Sealant with fixed stops at the

interior.

4.	Thermal Barrier:  Provide continuous thermal barrier by means of a poured and debridged pocket

consisting of a two-part, chemically curing high density polyurethane which is bonded to the

aluminum by YKK ThermaBond Plus®. Systems employing non structural thermal barriers are not

acceptable.

2.02 	MATERIALS

A.	Extrusions:  ASTM B 221 (ASTM B 221M), 6063-T5 Aluminum Alloy.

B.	Aluminum Sheet:

1.	Anodized Finish:  ASTM B 209 (ASTM B 209M), 5005-H14 Aluminum Alloy, 0.050" (1.27 mm)

minimum thickness.

2.	Painted Finish:  ASTM B 209 (ASTM B 209M), 3003-H14 Aluminum Alloy, 0.080" (1.95 mm)

minimum thickness.

2.03 ACCESSORIES

A.	Manufacturer's Standard Accessories:

1.	Fasteners:  Zinc plated steel concealed fasteners; Hardened aluminum alloys or AISI 300 series

stainless steel exposed fasteners.

2. 	Glazing:  Setting blocks, edge blocks, and spacers in accordance with ASTM C 864, shore durometer

hardness as recommended by manufacturer; Glazing gaskets in accordance with ASTM C 864.

3.	0.050 Aluminum Sill Flashing End Dams featuring 3 point attachment.

2.04	RELATED MATERIALS (Specified in Other Sections)

A.	Aluminum swing doors:  Section 08 41 13 35H Impact Resistant Heavy Duty Swing Doors.

2.05 	FABRICATION

A.	Shop Assembly:  Fabricate and assemble units with joints only at intersection of aluminum members

with hairline joints; rigidly secure, and sealed in accordance with manufacturer's recommendations.

2.06	FINISHES AND COLORS

A.  YKK AP America Anodized Plus® Finish:

CODE	DESCRIPTION

YS1N*	Clear Anodized Plus®

YH3N	Champagne Anodized Plus®

YB1N	Medium Bronze Anodized Plus®

YB5N*	Dark Bronze Anodized Plus®

YK1N*	Black Anodized Plus®

YW3N	White Anodized Plus®

M			Mill Finish

* Indicates standard finish usually carried as inventory.

Anodized Plus® is an advanced sealing technology that completely seals the anodic film yielding

superior durability (See AAMA 612).

B.	Anodized Finishing:  Prepare aluminum surfaces for specified finish; apply shop finish in accordance

with the following:

1. 	Anodic Coating:  Electrolytic color coating followed by an organic seal applied in accordance with

the requirements of AAMA 612. Aluminum extrusions shall be produced from quality controlled

billets meeting AA-6063-T5.

a. 	Exposed Surfaces shall be free of scratches and other serious blemishes.

b. 	Extrusions shall be given a caustic etch followed by an anodic oxide treatment and then sealed

with an organic coating applied with an electrodeposition process.

c. 	The anodized coating shall comply with all of the requirements of AAMA 612: Voluntary

Specifications, Performance Requirements and Test Procedures for Combined Coatings of Anodic

Oxide and Transparent Organic Coatings on Architectural Aluminum. Testing shall demonstrate

the ability of the finish to resist damage from mortar, salt spray, and chemicals commonly found

on construction sites, and to resist the loss of color and gloss.

d. 	Overall coating thickness for finishes shall be a minimum of 0.7 mils.

C.	High Performance Organic Coating Finish:

1.	Fluoropolymer Type:  Factory applied two-coat 70% Kynar resin by Arkema or 70% Hylar resin by

Solvay Solexis, fluoropolymer based coating system, Polyvinylidene Fluoride (PVF-2), applied in

accordance with YKK AP procedures and meeting AAMA 2605 specifications.

2.	Colors:  Selected by Architect from the following:

a.  	Standard coating color charts.

b.  Custom coating color charts.

c.  	Color Name and Number:

D.	Finishes Testing:

1.  Apply 0.5% solution NaOh, sodium hydroxide, to small area of finished sample area; leave in place

for sixty minutes; lightly wipe off NaOh. Do not clean area further.

2.  Submit samples with test area noted on each sample.

PART 3 EXECUTION

3.01 MANUFACTURER'S INSTRUCTIONS / RECOMMENDATIONS

A. 	Compliance:  Comply with manufacturer's product data, including product technical bulletins,

installation instructions, and product carton instructions. The latest installation instructions are available

at www.ykkap.com.

3.02	EXAMINATION

A.	Site Verification of Conditions:  Verify substrate conditions (which have been previously installed under

other sections) are acceptable for product installation in accordance with manufacturer's instructions.

3.03 	PREPARATION

A.	Adjacent Surfaces Protection:  Protect adjacent work areas and finish surfaces from damage during

product installation.

1.	Aluminum Surface Protection:  Protect aluminum surfaces from contact with lime, mortar, cement,

acids, and other harmful contaminants.

3.04	INSTALLATION

A.	General:  Install manufacturer's system in strict accordance with shop drawings, and within specified

tolerances.

1.	Shim and brace aluminum system before anchoring to structure.

2.	Provide sill flashing at exterior storefront systems. Extend extruded flashing continuous with splice

joints; set in continuous beads of sealant and wept.

3.	Verify storefront system allows water entering system to be collected in gutters and wept to exterior.

Verify metal joints are sealed in accordance with the manufacturer's instructions.

4.	Seal metal to metal storefront system joints using sealant recommended by system manufacturer.

3.05	FIELD QUALITY CONTROL

A. 	Manufacturer's Field Services:  Upon request, provide manufacturer's field service consisting of site visit

for inspection of product installation in accordance with manufacturer's instructions.

B. 	Field Test:  Conduct field test to determine watertightness of storefront system. Conduct test in

accordance with AAMA 501.2.

3.06	ADJUSTING AND CLEANING

A.	Adjusting:  Adjust operating items as recommended by manufacturer.

B.	Cleaning:  The General Contractor shall clean installed products in accordance with manufacturer's

instructions prior to Owner's acceptance, and remove construction debris from project site. Legally

dispose of debris.

C.	Protection:  The General Contractor shall protect the installed product's finish surfaces from damage

during construction.

END OF SECTION

This document supersedes all previous versions.